| Computer Organization and Architecture | SAVE AND SHARE WITH YOUR CONNECTIONS TO HELP THEM Created by:- Bhabani Shankar Pradhan (Linkedin) https://www.linkedin.com/in/bhabanishankarpradhan07/ |
| --- | --- |

Branch:- Computer Science and Engineering/ Information Technology

Subject :- Computer Organization and Architecture.

CO1 explain and illustrate the execution cycle of a program.

CO2 classify the components of digital computer and designing of control and memory

units with performance evaluation.

CO3 compute different type of binary arithmetic operation.

CO4 identify the characteristics of memories and explain memory and IO devices working

process.

MCQs (2 Marks)

Unit-I

- ______ bus structure is usually used to connect I/O devices.	CO 1,PO1
a) Single bus
b) Multiple bus
c) Star bus
d) Rambus

- The ALU makes use of _______ to store the intermediate results. CO 1,PO1
a) Accumulators
b) Registers
c) Heap
d) Stack

- The Input devices can send information to the processor. CO 1,PO1
a) When the SIN status flag is set
b) When the data arrives regardless of the SIN flag
c) Neither of the cases
d) Either of the cases

- The control unit controls other units by generating ____    CO 1,PO1
a) Control signals
b) Timing signals
c) Transfer signals
d) Command Signals

- ______ are numbers and encoded characters, generally used as operands. CO 1,PO1
a) Input
b) Data
c) Information
d) Stored Values

- The decoded instruction is stored in ______
a) IR
b) PC
c) Registers
d) MDR

- The instruction -> Add LOCA, R0 does _______			CO 1,PO1
a) Adds the value of LOCA to R0 and stores in the temp register
b) Adds the value of R0 to the address of LOCA
c) Adds the values of both LOCA and R0 and stores it in R0
d) Adds the value of LOCA with a value in accumulator and stores it in R0

- MAR stands for ___________						CO 1,PO1
a) Memory address register
b) Main address register
c) Main accessible register
d) Memory accessible register

- The sign magnitude representation of -1 is __________		CO 1,PO1
a) 0001
b) 1110
c) 1000
d) 1001

- The ALU gives the output of the operations and the output is stored in the ________

CO 1,PO1
a) Memory Devices
b) Registers
c) Flags
d) Output Unit

- The bitwise complement of 0 is ___________			CO 1,PO1
a) 00000001
b) 10000000
c) 11111111
d) 11111110

Unit-II

- Processor which is complex and expensive to produce is ________ CO 2,PO1
a) RISC
b) EPIC
c) CISC
d) Multi-core

- A circuitry that processes that responds to and processes the basic instructions that are required to drive a computer system is ________ CO 2,PO1
a) Memory
b) ALU
c) CU
d) Processor

- Consider the following statements and choose the correct option from list based on the statements.						CO 2,PO1

Statements:

- Content of IR

- content of control step counter

- content of external inputs

- content of condition code

The control unit generate control signals based on the inputs,

- Statement 1 and 2

- Statement 3 and 4

- Statement 1, 3 and 4

- All of these

- What is MFC stands for.				CO 2,PO1

- Master Fast Computing

- Memory Function Complete

- Memory Friction Component

- Mean Fraction Computing

- The area of components involves in execution of instruction refers to           CO 2,PO1

- executor

- fetcher

- datapath

- dataflow

- How many processors are there in a Illiac Machine:			          CO 2,PO1

- 16

- 42

- 64

- 32

- What is Data Routing Function.					CO 2,PO1

- defines fetching of data

- defines transferring of data from one PE to another PE

- defines fetching of data from PEM

- None of the above

- Data forwarding is the technique of 				CO 2,PO1

- avoid data hazard in pipeline

- avoid instruction hazard in pipeline

- avoid structural hazard in pipeline

- None of the above

- Instruction hazard in branch instruction can be avoided by ___________ CO 2,PO1

- delayed branch

- branch prediction

- both a) and b)

- None of the above

- Use of NOP in Instruction execution is 					CO 2,PO1

- Hardware solution for hazard

- Software solution for hazard

- Both of the above

- None of the above

Unit-III

- The carry generation function: ci + 1 = yici + xici + xiyi, is implemented in ____________

CO 2, PO1

a) Half adders

b) Full adders

c) Ripple adders

d) Fast adders

- Which option is true regarding the carry in the ripple adders?		CO 2, PO1

a) Are generated at the beginning only

b) Must travel through the configuration

c) Is generated at the end of each operation

d) None of the mentioned

- In a normal adder circuit, the delay obtained in a generation of the output is _______

CO 2, PO1

a) 2n + 2

b) 2n

c) n + 2

d) None of the mentioned

- The product of 1101 & 1011 is ______				CO 2, PO1

a) 10001111

b) 10101010

c) 11110000

d) 11001100

- We make use of ______ circuits to implement multiplication. CO 2, PO1

a) Flip flops

b) Combinatorial

c) Fast adders

d) None of the mentioned

- The method used to reduce the maximum number of summands by half is _______

CO 2, PO1

a) Fast multiplication

b) Bit-pair recording

c) Quick multiplication

d) None of the mentioned

- CSA stands for?						CO 2, PO1

a) Computer Speed Addition

b) Carry Save Addition

c) Computer Service Architecture

d) None of the mentioned

- In IEEE 32-bit representations, the mantissa of the fraction is said to occupy ______ bits.

CO 2, PO1

a) 24

b) 23

c) 20

d) 16

- In double precision format, the size of the mantissa is ______	CO 2, PO1

a) 32 bit

b) 52 bit

c) 64 bit

d) 72 bit

- In full adders the sum circuit is implemented using ________	CO 2, PO1

a) And & or gates

b) NAND gate

c) XOR

d) XNOR

Unit-IV

- The key factor/s in commercial success of a computer is/are ________ CO 4, PO1

a) Performance

b) Cost

c) Speed

d) Both Performance and Cost

- The duration between the read and the mfc signal is ______		CO 4, PO1

a) Access time

b) Latency

c) Delay

d) Cycle time

- The logical addresses generated by the cpu are mapped onto physical memory by ____________								CO 4, PO1

a) Relocation register

b) TLB

c) MMU

d) None of the mentioned

- The time taken to transfer a word of data to or from the memory is called as ______

CO 2, PO1

a) Access time

b) Cycle time

c) Memory latency

d) None of the mentioned

- Cache memory is implemented by using the technique is, 		CO 4, PO1

- reference of memory

- locality of reference

- both of the above

- none of the above

- Cache mapping function is 						CO 4, PO1

- defines the cache block which is going to accommodate main memory block

- defines the main memory block which is going to accommodate cache block

- both of the above

- none of the above

- The access time of memory is …………… the time required for performing any single CPU operation.								CO 4, PO1

a) Longer than

b) Shorter than

c) Negligible than

d) Same as

- Memory address refers to the successive memory words and the machine is called as …………								CO 4, PO1

a) word addressable

b) byte addressable

c) bit addressable

e) Terra byte addressable

- Memory interleaving is							CO 4, PO1

- fetching/storing data from/to a word in successive memory module

- fetching/storing data from/to successive memory word

- fetching/storing data from/to a successive word in successive memory module

- None of the above

- j mod k is used in 							CO 4, PO1

- set-associative mapping

- associative mapping

- direct mapping

- None of the above

Short answer type questions (2 Marks)

Unit-I

- Define Computer architecture and computer organization.	CO 1, PO1

- Give the different functional units of digital computer.		CO 1, PO1

- Define Amdhal’s law.						CO 1, PO1

- Write the addressing mode and instruction format used the instruction ADD (R3), R1

CO 1, PO 2

- Define Bus? What is the use of buffer register?				CO 1, PO 1

- Compare single bus structure and multiple bus structure?	CO 1, PO 2

- What is System Software? Give an example?			CO 1, PO 2

- What is processor execution time of a program?		CO 1, PO 1

- Write down the basic performance equation?			CO 1, PO 1

- List out the various registers are used in basic operational concept? CO 1, PO 1

Unit-II

- Define pipelining? Give an example of pipeline execution.	CO 3, PO 1

- Draw the block diagram of hardwired control unit.		CO 1, PO 2

- Give the various types of multiprocessor environment based on Flynn’s classification.     								CO 3, PO 1

- Illustrate the block diagram of a PE in Illiac machine.		CO 3, PO 1

- Write any to implementation of array processor.		CO 3, PO 1

- Explain single bus datapath.		x			CO 1, PO 2

- Write the difference between horizontal and vertical microprogrammed control. CO 1, PO 2

- Write the techniques to avoid instruction hazard in conditional branch instruction.		CO 4, PO 1

- Write the techniques to avoid instruction hazard in unconditional branch instruction.	CO 4, PO 1

- Write the techniques to avoid data hazard.					CO 4, PO 1

Unit-III

- Write the equation for sum and carry-out in full adder.			CO 1, PO 1

- Write the corresponding sign-and-magnitude, 1’s complement and 2’s complement representation of following numbers: 					CO 2, PO 1

- -10

- 8

- Illustrate the difference between booth’s bit-recording and booth’s bit-pair recording. CO 2, PO 1

- Find the quotient and remainder using restoring division of the following numbers.

16 ÷ 3									CO 2, PO 2

- Find the quotient and remainder using non-restoring division of the following numbers.

25 ÷ 4									CO 2, PO 2

- Represent the following numbers in single precision of IEEE format.

-3.65									CO 2, PO 2

- Represent the following numbers in double precision of IEEE format.

+8.25									CO 2, PO 2

- Perform the fast multiplication on the following 2’s complement numbers.

13 X -9									CO 2, PO 2

- Explain with an example the addition/subtraction rule of floating point number. CO 3, PO 2

- Write the difference between restoring and non-restoring binary division. CO 2, PO 2

Unit-IV

- Explain the memory hierarchy in modern computer. 			CO 3, PO 1

- Explain with neat diagram of static memory cell.			CO 4, PO 1

- Explain the different types of endianness implemented the computer system for storing of data in memory.							CO 2, PO 1

- Write about the following terms: 					CO 2, PO 1

- Byte-addressability

- Word alignment

- Write about the following terms: 					CO 2, PO 1

- Word address

- Byte location

- Define miss rate and miss penalty?					CO 2, PO 1

- Define locality of reference?						CO 2, PO 1

- What is the formula for calculating the average access time experienced by the processor?								CO 2, PO 1

- What is the formula for calculating the average access time experienced by the processor in a system with two levels of caches?			CO 2, PO 1

- What is replacement algorithm? 					CO 2, PO 1

- What is associative search?						CO 2, PO 1

Shor Answer Type Questions: 5 Marks

UNIT-I

- Draw and explain a Von-Neumann architecture.			CO 1, PO 1

- Write the functions of MAR, MDR, IR and PC.			CO 1, PO 1

- Draw a Single bus structure of a system. 			CO 1, PO 1

- Define a machine instruction. How does it differ from user program?	CO 1, PO 1

- Write down different types of Instruction set architecture.	CO 1, PO 1

- What is the difference between RISC and CISC?			CO 1, PO 2

- How is a performance of a system measured?			CO 2, PO 1

- Calculate CPI of a computer which need 200 Mhz frequency to operate if number of instruction of a program is 75 and each require 3 number of average steps to execute.

CO 2, PO 2

- Define Amdahl’s law with suitable example.			CO 1, PO 1

- Define addressing mode of an instruction with example.		CO 2, PO 1

- What are different types of instruction format in machine instruction? CO 1, PO 2

- Write about auto-increment addressing mode with example.	CO 1, PO 1

- Represent the following instructions in RTN.			CO 1, PO 2

  - Add R1, R2, R3

  - Store B

UNIT-II

- Write the difference between hardwired and micro-programmed control unit.

- Define pipeline architecture of a system.

- What are different hazards occurring in pipeline architecture.

- Define data hazard occur in a system.				CO 4, PO 1

- What are Flynn’s classification of a computer system?		CO 3, PO 2

- Differentiate UMA and NUMA architecture.			CO 2, PO 2

- How does multiprocessor system improve the performance of a system?	CO 2, PO 2

- What is SIMD processor? write one example of it.		CO 2, PO 2

- Write down the advantages of array processor.			CO 2, PO 2

- How can data hazard be avoided in programming?		CO 4, PO 1

UNIT-III

- Solve the following operations by using 2’s complement.		CO 2, PO 1

27-17=?

- Draw a n-bit ripple adder circuit.					CO 4, PO 1

- What is the number of gate delay for 16- bit carry look ahead adder by using 4-bit adders? CO 3, PO 1

- Represent -16.25   in single precision and double precision IEEE format.	CO 2, PO 1

- If a 32-bit IEEE format for a floating point number is B0050000 then find its decimal equivalent.									CO 2, PO 1

- Write the steps for multiplication of two floating point number.			CO 2, PO 1

- What are the advantages of Booth algorithm?					CO 2, PO 2

- How does a fast multiplication method reduce number of gate delay?		CO 3, PO 1

- Between restoring and non-restoring division algorithm which perform faster and why? CO 2, PO 2

- Find the product of the following 2’s complement numbers using booth’s multiplication.

CO 2, PO 2

- -15 X 25

- 32 X -16

UNIT-IV

- What is difference between SRAM and DRAM?

- Draw the memory hierarchy structure of a computing device.

- What is memory latency time?

- Define locality of reference property of cache memory.

- How does big endian byte addressability used in memory addressing system? Give one example

- How does little endian byte addressability used in memory addressing system? Give one example

- Write the memory alignment address of a 32- bit processor.

- What is write through and write back protocol in cache memory?

- Write different cache mapping functions used in memory.

- For a 120 cache memory references if hit ratio is 0.75, cache access time is 25ns , miss penalty is 100ns then find the cache performance.

- Why is virtual memory required in system?

- What is DMA?

- What is the necessity of Interrupts?

Long Question (10 Marks)

Unit-I

- Write the assembly language notation and corresponding RTN for the following expression, if the system is using single accumulator processor and the system using stack operation.						(CO 2, PO 2)

Z = (A + B) X (C + B)

E = (A X B) + (C X D)

- Explain different bus structure with neat block diagram. (CO 1, PO 1)

- Perform all the type of the shift operation on the data in the register. 	(CO 2, PO2)

R0 = 10000111110110

R1 = 101010101010101

- Classify addressing modes and explain each type with suitable example. (CO 1, PO2)

- Explain different types of instruction execution and sequencing.	(CO 1, PO2)

- Explain with suitable example the various ways of representing signed number in computer and give an account of comparison among them.		 (CO 1, PO2)

Unit-II

- Explain with neat diagram single bus data-path in a processor.	(CO 1, PO2)

- Write the control sequence step for the following instructions,		(CO 1, PO2)

      - ADD (R3), R1

      - MUL (R1), R2, (R3)

- Explain with neat diagram multi-bus data path in a processor.		(CO 1, PO2)

- Explain horizontal and vertical microprogrammed control with suitable example. (CO 1, PO2)

- Explain data forwarding with suitable example for avoiding the data hazard in pipeline execution.								(CO 4, PO2)

- Explain the data path in pipeline execution.				(CO 4, PO2)

- Explain the execution cycle for instruction execution with example.	(CO 1, PO2)

- Write the various techniques to avoid the instruction hazard in pipeline execution. (CO 4, PO2)

- Define multiprocessor computing? Explain with suitable example for various types of multiprocessor machine according to Flynn’s classification.		(CO 4, PO2)

- A non-pipeline system takes 50 ns to process a task. The same task can be processed in a 6 segment pipeline with a clock cycle of 10 ns. Determine the speed up ratio of the pipeline for 100 tasks. What is the maximum speed up that can be achieved?

Unit-III

- Draw the flowchart for Booth’s algorithm for multiplication of signed 2’s complement numbers and explain with an example.

- Explain with a diagram the designs of a fast multiplier using carry save Adder circuit. Give the block diagram for a floating-point adder / subtractor unit and discuss its operation.

- a) Explain the floating point addition and subtraction 8M
b) State the Non – restoring division technique 7M

- Explain Division techniques with an example

- Design an arithmetic circuit with one selection variable S and two n-bit data inputs A & B. The circuit generates the following four arithmetic operations in conjunction with the input carry Cin. Draw the logic diagram for the first two stages.

- Multiple (-7)10 with (3)10 by using Booth’s multiplication. Give the flow table of the multiplication.

- Explain the Hardware implementation of multiplication in signed-magnitude data? Using this multiply two numbers (Multiplicand is 23 and Multiplier is 19)?

Unit-IV

- a) With a neat diagram, describe DMA transfer in a computer system 10M

b) Describe in detail about associative memory 5M

- Discuss the different mapping techniques used in cache memories and their relative merits and demerits. 15M

- Assume a cache miss penalty is 100 clock cycles, and all instructions take 1.0 clock cycles. Let the average miss rate is 2%, there is an average of 1.5 memory references per instructions, and the average number of cache misses per 1000 instructions is 30. What is the impact on the performance and calculate the impact using both misses per instruction and miss rate?

- Illustrate the characteristics of some common memory technologies 15M

- What is cache memory and explain different cache memory mapping techniques 15M

- a) Describe virtual memory in detail. 7M

b) Compare and contrast between Asynchronous DRAM and Synchronous DRAM. 8M

- a) What is an interrupt and explain different types of Interrupts.

- What do you mean by Speed-Up of pipeline? Derive equations of Speed-Up and Efficiency for Pipeline, Super pipeline and Super scalar architecture.

- What is virtual memory? How a logical address is mapped to physical address in virtual concept? Explain with example and diagram